The Pfa Reachability Bound: How Threshold-Offset Growth Rate
Limits Achievable False-Alarm Calibration in Parametric SAR CFAR Detectors

[Author Name]

[Department / Institution]

[email address]

Abstract—Constant-false-alarm-rate (CFAR) detectors for synthetic aperture radar (SAR) ship detection are designed to deliver a requested false-alarm probability regardless of clutter level. This paper identifies a source of achieved-versus-nominal Pfa divergence that is independent of clutter-model mismatch, finite-sample estimation noise, and cross-scene variation -- all previously documented mechanisms -- and is instead a property of the clutter distribution family itself. Every detector considered here reduces to a shared decision rule in which a threshold offset delta(shape, Pfa) is added to a clutter-level term; how fast that offset grows as the requested Pfa shrinks toward zero is fixed by the family's tail type. We derive this growth rate for six clutter families used in SAR CFAR detection (Weibull, Lognormal, Generalized Gamma, G0, Burr Type-XII, and K-distribution) -- two exactly and four as verified asymptotics, each cross-checked numerically against its real implementation -- and show they separate into three classes set by the extreme-value type of the clutter tail: log-logarithmic, square-root-logarithmic, and linear growth. We further resolve an apparent collision with a recently published cross-scene Pfa-divergence result, showing on a controlled, fixed-scene grid that the two divergence axes are empirically distinct, and attach exact Clopper-Pearson confidence intervals -- computed from the clutter-window sample size rather than the sparse ship count -- to every reported figure, revealing that large, alarming divergence numbers are typically well-supported while small, reassuring calibration numbers frequently are not. No offset function derived here is bounded above; the practical consequence is not an unreachable Pfa floor but a reachability bound -- a point past which further tightening the nominal Pfa produces changes too small to survive ordinary estimation noise or fixed-point quantization.

Index Terms—Constant false alarm rate (CFAR), synthetic aperture radar (SAR), false alarm probability calibration, clutter modeling, Weibull distribution, K-distribution, Generalized Gamma distribution, extreme value theory, statistical hypothesis testing, radar detection theory.

I. INTRODUCTION / THESIS

A constant-false-alarm-rate (CFAR) detector's defining promise is that the operator's requested false-alarm probability, the nominal Pfa, is what the system actually delivers, regardless of clutter level or clutter type. In practice this promise is known to be imperfect. Recent work on SAR ship detection ([1] reports substantial gaps between requested and achieved Pfa across scenes, generally attributed to clutter- model mismatch and scene-to-scene statistical diversity — the achieved rate drifts because the scene the detector is pointed at does not match the scene it was calibrated on. This is a real and well-documented failure mode, and §3 of this paper shows directly that it is not the mechanism this paper is about.

This paper identifies a second, model-internal mechanism, orthogonal to scene mismatch: even for a single scene, a single window geometry, and a clutter model that is exactly correctly specified with noiseless shape estimates, the achieved Pfa can still diverge from the nominal one by several orders of magnitude — because of how the detector's own threshold-offset function is shaped, not because of anything wrong with the clutter fit. Every detector considered here reduces to the same decision rule, T_log = c1 + delta(shape, Pfa), where c1 carries the clutter level and delta is a function of the fitted shape parameter(s) and the requested Pfa alone. The question this paper asks is simple and, as far as a search of the accessible literature could establish (§5), not previously asked in these terms: how fast does `delta` grow as `Pfa -> 0`, and does that growth rate differ enough between commonly used SAR clutter families to matter in practice?

It differs enormously. §2 derives — exactly for two of six families, as a verified asymptotic for the other four, every derivation checked numerically against the real implementation rather than trusted as algebra — the Pfa -> 0 growth rate of all six clutter families this project's broader comparison uses (Weibull, Lognormal, Generalized Gamma, G0, Burr XII, K-distribution). They separate cleanly into three classes, and the class a family falls into is set by the extreme-value type of its clutter tail, not by which specific detector it happens to be: light, Gumbel-domain tails (Weibull, K-distribution, and half of Generalized Gamma's parameter range) give a threshold offset that grows only as log log(1/Pfa); heavier, power-law tails (Burr XII, G0, and the other half of Generalized Gamma) give a threshold offset that grows a full order faster, as log(1/Pfa).

To be precise about what this growth-rate result does and does not claim, stated here rather than left for a reader to infer: no detector's threshold offset is bounded above. Every one of the six formulas derived in §2 diverges to infinity as Pfa -> 0 — there is no asymptotic floor on the achievable false-alarm rate, and this paper does not claim one exists. What differs between the three growth classes is how much a given decrease in the nominal Pfa dial actually moves the threshold. For a log-log-growing family, asking for a nominal Pfa ten orders of magnitude smaller moves the threshold offset by only a few units of delta — an amount easily swamped by ordinary shape-estimation noise (already documented, for real SAR data, as this project's own F9 finding) or by the quantization step of a realistic fixed-point hardware pipeline. The dial still turns; it simply stops doing anything useful long before it reaches the requested value. This paper calls that operational fact — not an asymptotic bound on achievable Pfa, but a practical bound on how far the nominal-Pfa dial can be turned before its effect disappears into noise — the Pfa reachability bound.

This paper's contributions are:

1) A closed-form or tightly-bounded-asymptotic derivation of the Pfa -> 0 growth rate for all six SAR clutter families in this comparison, generalizing a previously known two-family result (Weibull, Lognormal) into a three-class taxonomy tied directly to the extreme-value type of the underlying clutter tail — a theorem about distribution families, not a pairwise curiosity (§2).

2) Direct resolution of an apparent collision with a recently published cross-scene Pfa-divergence result, showing on a controlled, fixed-scene, swept-window-size grid that this paper's divergence axis is empirically distinct from that one (§3).

3) Exact Clopper-Pearson confidence intervals, computed from the clutter- window sample size rather than the much sparser ship count, attached to every reported divergence figure — and a demonstration that this precision is fundamentally asymmetric: large, alarming divergence numbers are usually well-supported by the data, while small, reassuring "well-calibrated" numbers frequently are not (§4).

4) An audited, though not exhaustive, search of the accessible IEEE literature establishing that this specific growth-rate mechanism does not appear to have been identified previously under different terminology (§5).

Section 2 develops the growth-rate taxonomy and its numerical verification. Section 3 resolves the collision with prior cross-scene work. Section 4 attaches statistical rigor to every headline number this paper and its companion measurements report. Section 5 reports the literature search. Section 6 discusses what the growth-rate result does and does not imply for detector selection and for how this class of result should be reported generally, and Section 7 concludes.

II. CLOSED-FORM GROWTH RATES

Verified, pre-existing (this project's F9/F10 findings):

• Weibull: K(Pfa) = gamma + log(-log Pfa) — grows log-logarithmically.

• Lognormal: z(Pfa) ~ sqrt(log(1/Pfa)) — grows as a square root of log-inverse-Pfa, structurally slower-diverging than Weibull's in the relevant sense the roadmap's thesis rests on. F10 additionally proves Weibull and Lognormal are the SAME decision rule up to one Pfa-dependent scalar (delta_Lognormal/delta_Weibull constant to 4e-16/6.7e-16 across a decade of c2, at Pfa=1e-3 and 1e-6 respectively) — they differ only through that scalar and their clamp behavior, not through independently free growth rates.

A. The remaining four (2026-09, roadmap item 2)

Method. Each of the four remaining detectors' delta(shape, Pfa) = T_log - c1 already has an EXACT implicit form in its *CFAR_TLog.m (an inverse incomplete gamma, inverse incomplete beta, or — for K — a numerically solved 1-D mixture integral; see each file's header). What's new here is each one's closed-form or tightly-bounded asymptotic behavior AS Pfa -> 0, derived by standard tail-inversion analysis (large- and small-argument series of the incomplete gamma/beta functions; Laplace's-method saddle-point analysis for K's mixture integral, which has no elementary tail series at all). Every derivation below was cross-checked numerically against the real *CFAR_TLog.m output — not just algebra on paper — via _comparison/verify_growth_rates.m, at both this project's real sweep Pfa range (1e-1...1e-6) and much deeper (1e-9...1e-40), confirming the error between the derived formula and the real code shrinks as Pfa -> 0. This check caught one real sign error before it reached this document (BUG_LOG D25: the Generalized Gamma v<0 branch's leading term had the wrong sign in an early draft — predicted values came out negated relative to the real code, an unmistakable tell, fixed and reverified).

Result — the six distributions fall into three growth classes, and the class is set by the underlying tail TYPE, not by which detector it happens to be:

| Class | Growth as Pfa -> 0 | Members |
| --- | --- | --- |
| Log-log (slowest) | ~ log log(1/Pfa) | Weibull (exact); GenGamma v>0 branch (asymptotic); K-distribution (asymptotic) |
| Sqrt-log | ~ sqrt(log(1/Pfa)) | Lognormal (asymptotic) |
| Linear-log (fastest) | ~ log(1/Pfa) | Burr XII (exact); G0 (asymptotic); GenGamma v<0 branch (asymptotic) |

Generalized Gamma (GenGammaCFAR_TLog.m: delta = [log(G) - psi(k)]/v, G a gamma-quantile whose tail depends on the sign of v):

v > 0 (upper gamma tail):  delta ~ [ log log(1/Pfa) - psi(k) ] / v
                                        + O( log log(1/Pfa) / (v * log(1/Pfa)) )
    v < 0 (lower gamma tail):  delta ~ [ log(1/Pfa)/k - log(Gamma(k+1))/k + psi(k) ] / |v|
                                        + O( Pfa^(1/k) / |v| )

The two branches of the SAME family land in different growth classes: v>0 inherits Weibull's exact log-log form (an upper incomplete-gamma tail decays like x^(k-1) e^-x, the same Gumbel-type exponential tail that gives Weibull its own log-log growth — k=1 reduces the two formulas to be identical, confirmed to 0.000 absolute error at every Pfa tested); v<0 inherits Burr/G0's fast linear-log form (a lower incomplete-gamma tail near 0 decays like x^k, a power law, the same tail type that gives Burr and G0 their linear growth). This asymmetry is real, not a labeling convenience — the v<0 branch converges to its asymptotic form polynomially fast (O(Pfa^(1/k)), machine precision by Pfa=1e-12 in the numerical check), while the v>0 branch converges only logarithmically slowly (O(log L / L), still ~0.05–0.2 absolute error even at Pfa=1e-40 for the k values tested) — a second, independent piece of evidence (beyond F9's already-documented 1/v noise amplification) that Generalized Gamma's worst-case calibration behavior concentrates on the positive-skew branch.

G0 (G0CFAR_TLog.m: delta = 0.5*[psi(u)-psi(L) + log(x/(1-x))], x solving the regularized incomplete beta I_{1-x}(u,L) = Pfa):

delta ~ 0.5*(psi(u) - psi(L)) + [ log(1/Pfa) - log(u * B(u,L)) ] / (2u)

using the small-argument beta series I_y(u,L) ~ y^u / (u B(u,L)) as y -> 0. Error O(Pfa^(1/u)) — already at 5e-7 absolute error by Pfa=1e-6 for u=1, machine precision by Pfa=1e-20. Linear-log class, rate 1/(2u) — SMALLER u (rougher texture, heavier tail) grows the offset FASTER, i.e. calibrates tighter; larger u degrades toward the slower classes' behavior, consistent with G0's real-data ratio (31.8x at Pfa=1e-6, F9) sitting well below Weibull's/GenGamma's but not as low as a u<<1 asymptotic optimum would suggest.

Burr XII (BurrCFAR_TLog.m: delta = [psi(kappa)-psi(1) + log(Pfa^(-1/kappa)-1)]/rho) — this one is genuinely exact, not merely asymptotic, because Burr's survival function is algebraically invertible in closed form (like Weibull's):

delta = [ psi(kappa) - psi(1) + log(1/Pfa)/kappa + log1p(-Pfa^(1/kappa)) ] / rho

the log1p term is O(Pfa^(1/kappa)) — literally exponentially small in log(1/Pfa)/kappa, not merely "eventually negligible." Linear-log class, rate 1/(kappa*rho). Verified: matches the real BurrCFAR_TLog.m to 1e-4–1e-1 absolute error even at Pfa=1e-1 and to double-precision floor by Pfa=1e-20.

K-distribution (KCFAR_TLog.m: delta = 0.5*log(x_pfa) - kappa1(a), x_pfa solving the mixture survival integral S(x;a)=E_tau[exp(-x/tau)]= Pfa, tau ~ Gamma(a,1/a)) — no elementary tail series exists (this is exactly why the implementation falls back to fzero on a numerical integral rather than an incomplete-function inverse), so this derivation uses Laplace's-method saddle-point analysis directly on the mixture integral. Maximizing the integrand's exponent -x/tau - a*tau over tau gives a saddle at tau* = sqrt(x/a), exponent value -2*sqrt(a*x) — the well-known stretched-exponential K-distribution tail (equivalently, the large-argument asymptotic of the modified Bessel function K_{a-1} that appears in K's closed-form density). Inverting S(x)=Pfa through this saddle point:

x_pfa       ~ log(1/Pfa)^2 / (4a)
    delta_K(Pfa) ~ log log(1/Pfa) - 0.5*log(4a) - kappa1(a)

Log-log class — the SAME slow growth rate as Weibull, independent of a to leading order (verified: at a=0.5, the formula already matches the real KCFAR_TLog.m mixture-integral solve to 1.5e-5 absolute error across the entire tested range Pfa=1e-1...1e-20; at larger a, convergence is the same O(log L / L) slow rate as GenGamma's v>0 branch — structurally expected, since both derivations are large-argument gamma-type tail asymptotics). This gives K-distribution CFAR a second, independent disadvantage on top of F13's already-documented support-condition collapse: even where K's c2 > pi^2/24 condition IS met, its own threshold offset grows only as slowly in Pfa as Weibull's — with no 1/v- or 1/rho-style shape-dependent acceleration available to offset it, since the log-log term's coefficient here is fixed at 1, not divided by an estimated shape parameter the way GenGamma's/Burr's/G0's are.

Important caveat for the paper's own thesis: the growth-rate CLASS of a detector's delta(shape, Pfa) formula is a property of the assumed clutter-model FAMILY, holding shape fixed — it is not, by itself, a prediction of real-data calibration performance (F9's measured achieved/nominal ratio). The two can and do diverge: F13's GammaTex-MAP7 result (two-stage despeckle + plain-Gamma CFAR) sits in the SAME slow log-log class as GenGamma's v>0 branch and K (its formula, delta = 0.5*(log(gammaincinv(1-Pfa,a)) - psi(a)), is the v=2 special case of the v>0 branch's structure) — yet it achieved the tightest real-data calibration of any detector in this whole project (PfaRatio 0.83–0.97x on HRSID). The resolution is not a contradiction: growth rate governs how much a FIXED, correctly-specified model's own threshold moves as you ask for a smaller nominal Pfa; F9's real-data ratio is additionally governed by how well the fitted model's assumed tail matches the TRUE clutter tail at extreme quantiles, and GammaTex-MAP7's despeckled texture matches its assumed Gamma model far better than any of the six raw-data detectors match theirs. The paper's thesis should therefore be stated precisely as: growth rate sets a MODEL-INTERNAL reachability bound — how far the nominal-Pfa dial can practically move the achieved threshold at all, for a detector fit to data its own family actually describes — not a claim that growth rate alone ranks real-world calibration accuracy across mismatched-model detectors. That second effect (model/tail mismatch) is F9's separate, already-documented contribution, and the two should be presented as compounding, not identical, mechanisms.

Reproduce: _comparison/verify_growth_rates.m (new file, this session) — run via cfar_setup(); verify_growth_rates(); compares every formula above against the real GenGammaCFAR_TLog.m / G0CFAR_TLog.m / BurrCFAR_TLog.m / KCFAR_TLog.m output at representative shape values spanning the ranges this project's real sweeps report (see MedianShape/IQRShape columns in comparison_summary_*.csv), at Pfa from 1e-1 down to 1e-40.

III. RESOLVING AN APPARENT COLLISION WITH PRIOR CROSS-SCENE WORK

Resolved 2026-09, by direct query of existing data — no new sweep needed.

The question: [1] reports a cross-scene design-Pfa shift with window geometry FIXED per scene. This project's own F9 finding reports up to 3,162× measured-vs-nominal Pfa divergence at a FIXED nominal setting. Are these the same axis (making this project's number a restatement of [1]'s result) or different axes (making it a distinct, citable finding)?

Method: query comparison_summary_hrsid_full.csv and comparison_summary_ssdd_full_fixed60.csv (each: 60 images, HELD FIXED across every sli in {11,15,17,21,31,41,51,61,71,81,91,101,111,121,131, 141,151}) for whether the measured/nominal Pfa ratio (PfaRatio) at a fixed nominal Pfa changes materially as sli alone varies — the image/scene set never changes across the sweep, so any variation found is attributable to window geometry alone, not to [1]'s cross-scene mechanism.

Result — it does, substantially, on both datasets:

| Detector | HRSID fold-change (min→max ratio, Pfa=1e-6) | SSDD fold-change (Pfa=1e-6) |
| --- | --- | --- |
| Weibull | 1.5x (489→744) | 1.9x (848→1611) |
| Lognormal | 16.9x (1.2→20.0) | 38.9x (11.4→443) |
| GenGamma | 3.6x (351→1267) | 2.8x (1840→5060) |
| G0 | 5.0x (28→144) | 7.6x (20→150) |
| BurrXII | 4.8x (10→49) | 2.1x (40→86) |

Every detector on both datasets shows a substantial ratio swing driven by sli ALONE, with the image set held fixed throughout. Lognormal's SSDD swing (38.9x) is the largest — notable since F10 shows Lognormal is otherwise the mildest-growing/best-calibrated of the two exactly-derived detectors, underscoring that this window-size sensitivity is a real, separate mechanism from the growth-rate story in §2, not a restatement of it.

Reconciling the exact "3,162×" figure. That number is FINDINGS.md's original F9 table value: SSDD, sli=21, pooled over 100 images (a DIFFERENT, smaller/differently-sampled image set than the 60-image sweep above). Querying the 60-image sweep at the identical sli=21 gives GenGamma's ratio = 3,027.6x — a 4% difference from 3,162x, consistent with ordinary sample-to-sample variation between a 100-image and a 60-image draw, not a contradiction. The headline number is reproducible within expected noise; more importantly, it is NOT the ceiling — the 60-image SSDD sweep finds GenGamma's ratio reaches 5,060x at sli=151, higher than the original headline figure, entirely from window size at fixed Pfa on the same scenes.

Conclusion: BC-3 resolves in the paper's favor. The divergence axis this paper measures (achieved-vs-nominal at fixed nominal Pfa, window size swept, scene set fixed) is demonstrably distinct from [1]'s axis (cross-scene design-Pfa shift, window geometry fixed per scene) — moving sli alone, with the scene held constant, moves the divergence ratio by up to 38.9x. State this explicitly in the paper as the answer to the BC-3 falsifier, per the roadmap's item 1 instruction, converting what could have been a defensive footnote into an offensive one.

Reproduce: _comparison/Results/comparison_summary_hrsid_full.csv, comparison_summary_ssdd_full_fixed60.csv, both already in the repo from the earlier Phase 6.2a SLI-plateau work; no new sweep was required for this section.

IV. STATISTICAL RIGOR

Computed 2026-09 (roadmap item 4), from the clutter-WINDOW count (`BackgroundPixels`, order 1e5–4e7 per sweep cell), never the ship count — conflating the two was explicitly flagged as the most obvious small-sample objection a reviewer would raise, and is avoided here by construction.

Method. Each (detector, sli, Pfa) cell in the existing sweeps already pools FalsePixels (successes) out of BackgroundPixels (trials) — an exact 95% Clopper-Pearson interval is computed directly from these via the incomplete-beta identity (betaincinv, no toolbox needed — the same function G0's own threshold derivation uses), attached as new Pfa_CI_Lo/Pfa_CI_Hi/Ratio_CI_Lo/Ratio_CI_Hi columns (_comparison/compute_clopper_pearson.m, output comparison_summary_*_ci.csv). Caveat, stated rather than hidden: this treats each background pixel as an independent Bernoulli trial. Neighbouring reference-window pixels are not independent (overlapping windows, spatially correlated clutter), so the true interval is somewhat wider than the i.i.d. interval computed here — the same approximation implicit in using the raw pixel count as "the" sample size at all. A tighter treatment would need an effective-sample-size correction for spatial correlation; not attempted here, flagged as a limitation.

Result — the headline divergence numbers are tight; the well-calibrated ones are not, and that asymmetry is itself worth stating.

| Figure | PfaRatio | 95% CI | Relative width |
| --- | --- | --- | --- |
| GenGamma, SSDD, sli=21, Pfa=1e-6 (original F9 headline, 100 images) | 3,162.1x | [3,133.9x, 3,190.5x] | 1.8% |
| GenGamma, SSDD, sli=151, Pfa=1e-6 (§3's reconciled ceiling, 60 images) | 5,060.2x | [5,014.6x, 5,106.1x] | 1.8% |
| Weibull, SSDD, sli=51, Pfa=1e-6 | 978.6x | [958.7x, 998.8x] | 4.1% |
| Weibull, HRSID, sli=51, Pfa=1e-6 | 547.6x | [540.2x, 555.1x] | 2.7% |
| Lognormal, HRSID, sli=51, Pfa=1e-6 | 1.9x | [1.5x, 2.4x] | 47.4% |

Both headline GenGamma figures — the ones this paper's thesis leans on — are tightly bounded (<2% relative width) at their reported sample size, not a coincidence: they come from cells with tens of thousands of false pixels out of ~10-15M background pixels, plenty of statistical power. The detector this project already flags as best-calibrated (Lognormal, F9/F10) has the widest relative interval of the five — not because its measurement is worse, but because being well-calibrated means observing very few false pixels, and a small numerator inflates relative uncertainty even with millions of trials (73 false pixels here). This is worth stating explicitly in the paper: a tight "detector X is well-calibrated" claim needs a correspondingly larger sample than a "detector Y is badly miscalibrated" claim does, at the same nominal Pfa — the two kinds of claim are not symmetric in how much data they need to support.

A sharper example of the same asymmetry — zero observed false alarms is not proof of good calibration. Several G0 cells on the SSDD full grid report FalsePixels=0 at Pfa=1e-6 (e.g. sli=41: 0 false pixels out of 358,576 background pixels), giving a naive point-estimate ratio of exactly 0.00x — read uncritically, this looks like G0 is better than perfectly calibrated. The Clopper-Pearson interval corrects this immediately: the 95% upper bound is 10.3x nominal, not 0 — a textbook "rule of three" result (with zero successes in N trials, the 95% upper bound on the true rate is ~3/N). Zero observed false alarms at this sample size is consistent with anywhere from perfect calibration to a ratio an order of magnitude worse; it does not distinguish between them. Any future claim in this paper (or a downstream one) that a detector "never" produces a false alarm at some Pfa/sli combination must report the CI upper bound, not the raw zero.

Grid-wide distribution of relative CI width (SSDD full grid, 612 cells): median 7.9%, 90th percentile 67.7% — most cells are tightly bounded, but a meaningful tail (mostly G0/K cells at small sli, where the reference window is small and the nominal Pfa is already tight, so few false pixels accumulate) carries wide intervals. Any single-cell number pulled from these sweeps for a table or figure should be checked against its own CI column before being reported as precise.

Reproduce: _comparison/compute_clopper_pearson.m; outputs Results/comparison_summary_{main,ssdd_full_fixed60,hrsid_full}_ci.csv.

Addendum, confirmed 2026-09 (roadmap item 5) — the HRSID sample was expanded from 60 to 250 images (same seed=42 draw superset, same 17-`sli` grid), and the result independently validates the CI-width finding above rather than just repeating it. The large divergence figures are stable: GenGamma's ratio at every sli moved by at most ~5% (e.g. sli=51, Pfa=1e-6: 389.2x at 60 images -> 410.9x at 250 images; sli=11: 1267.2x -> 1315.5x), and Weibull similarly (547.6x -> 529.8x). Lognormal's small ratio, by contrast, moved by more than 2x (1.9x -> 4.2x at the same cell) — not a contradiction, but exactly what a wide relative CI predicts: a well-calibrated detector's point estimate is expected to swing substantially between independent samples of this size, while a badly-miscalibrated one's does not. This is real, out-of-sample confirmation of the CI-width argument above, not a restatement of it. Conclusion for the paper: report the divergence headline figures (GenGamma, Weibull) as stable and reproducible under sample expansion; report Lognormal's own ratio only alongside its CI, never as a bare point figure, since the point estimate alone is shown here to be sample-size-sensitive in a way the paper's own statistical-rigor section already predicted.

Reproduce: run_comparison_hrsid('NumImages',250,'Sli',[11 15 17 21 31 41 51 61 71 81 91 101 111 121 131 141 151],'Random',true,'Seed',42,'Tag','_hrsid_full250'); output Results/comparison_summary_hrsid_full250.csv.

V. RELATED WORK / PRIOR ART

Searched 2026-09 (roadmap item 6), via IEEE Xplore through the supervisor's institutional access (VIT Chennai MyLOFT portal) — queries covering the two alternate-terminology guesses ("CFAR loss saturation", "Pfa control breakdown"), achieved-vs-nominal Pfa divergence directly, small-sample/Monte-Carlo calibration, heavy-tailed order-statistics CFAR, and the supervisor's own group (Mahapatra) for any prior touch on this exact question.

Result: the search came back clean — no collision found. Seven papers were pulled and read in full or in relevant part; none states this project's reachability-bound thesis (that a detector's own δ(shape, Pfa) growth rate as Pfa -> 0 bounds how far the achieved Pfa can practically track the nominal one) under any terminology, alternate or otherwise:

| Paper | What it actually covers | Why it doesn't collide |
| --- | --- | --- |
| [2] Sarma & Tufts (2013) | UMP nonparametric rank-order CFAR, exact ROC bounds vs. CA-CFAR via complete enumeration | Different detector class entirely — rank statistics have an exactly enumerable threshold distribution (no MoLC shape-fitting, no asymptotic tail-quantile problem); "performance bounds" here means ROC curves, not a Pfa-reachability limit |
| [3] Redjem & Sahed (2025) | Exact closed-form Pfa for geometric-mean CFAR, monopulse and multipulse | Derives an exact Pfa formula (via Gamma/Exponential duality) but never asks how the threshold offset grows as Pfa shrinks — no achieved-vs-nominal divergence framing at all |
| [4] Bouchelaghem et al. (2025) | Min-rule combining CA-CFAR and ACOSD-CFAR thresholds for clutter-edge/interferer robustness | Solves spatial non-homogeneity (clutter edges, interfering targets), not Pfa calibration as a function of the nominal Pfa dial |
| [5] Hinz et al. (2012) | Doppler-domain background segmentation for surface-wave radar | Different application (HF ocean surveillance), different problem (clutter-edge avoidance) |
| [6] Iinatti (2000) | Spread-spectrum code-acquisition threshold/mean-acquisition-time tradeoffs | Different domain entirely (comms synchronization, Marcum-Q/Rician thresholds) — a keyword false-positive on "threshold setting principles," not SAR/CFAR clutter literature |
| [7] Liu et al. (2015) | Array/beamformer GLRT tunable detectors (KGLRT, AMF, ACE) for steering-vector mismatch | Multichannel array detection theory, unrelated to single-channel clutter-model Pfa calibration |
| [8] Petrović et al. (2025) | Parameterizable Chisel HDL generator emitting FFT+2D-CFAR pipelines | Not applicable to THIS paper (no Pfa-calibration content) — relevant instead to Paper 2/3 as a related-work citation for the hardware-generator angle |

This absence is reported as an audited search, not silence (per the roadmap's own instruction): the search was not IEEE-Xplore-exhaustive (five targeted query strings, not a systematic review), so it does not prove no such paper exists anywhere — but it is real evidence, not an assumption, that the specific "CFAR loss saturation" / "Pfa control breakdown" terminology guesses did not surface anything, and that the broader achieved-vs-nominal-Pfa-divergence angle did not either. This paper's prior-art risk from this specific pre-emption route is therefore lower than it was before this search, though not eliminated.

Reproduce: IEEE Xplore, institutional access, queries: "CFAR" AND "false alarm" AND ("saturation" OR "breakdown"); "CFAR" AND "threshold" AND "achieved" AND "nominal" AND "false alarm"; "CFAR" AND "calibration" AND "small sample" AND "Monte Carlo"; "order statistics" AND "CFAR" AND "Pfa" AND "heavy tail"; Mahapatra AND CFAR AND Pfa.

VI. DISCUSSION

The mechanism comes first; the numbers are its consequence, not the finding itself. It is tempting to lead with the largest measured divergence in this project's grid — Generalized Gamma reaching over 5,000x nominal Pfa at sli=151 on SSDD (§3, §4) — because it is the most dramatic single figure available. That framing is exactly backwards for this paper's purpose, and is avoided throughout: the number is a consequence of Generalized Gamma's v>0 branch sharing Weibull's log-log growth class while additionally dividing by an independently noisy estimated parameter v (§2.1); it is not itself the discovery. A reader who remembers only "5,060x" and not "log-log growth, divided by a noisy v" has learned a dataset-specific curiosity, not a distribution-family property — and, restated once more so it cannot be read as implying otherwise: that number is not a ceiling. Every delta(shape, Pfa) formula derived in §2 is unbounded; 5,060x is simply how far this particular grid's window sizes happened to push it. A larger sli range, or a different scene, would find it larger still, for the same underlying reason and with no asymptotic limit approached.

Growth-rate class and real-world calibration accuracy are related but not identical, and §2.1 already states the precise relationship rather than leaving it to be inferred. The clearest illustration is the GammaTex-MAP7 result reported elsewhere in this research program (a two-stage despeckle-then-Gamma-CFAR pipeline): its own delta formula sits in the same slow log-log class as Weibull and K-distribution, yet it achieves the tightest real-data calibration of any detector measured in this comparison. This is not a counterexample to the growth-rate result — it is exactly what the result predicts once the two effects it depends on are separated. Growth rate is a property of a correctly specified model: it says how much the threshold moves as the nominal Pfa dial turns, holding the fitted shape fixed and assuming that shape genuinely describes the data. Real-world calibration accuracy (this project's own F9 finding) additionally depends on whether the fitted model's assumed tail matches the true clutter tail at the extreme quantiles a small Pfa probes. GammaTex-MAP7's despeckled texture happens to match its assumed Gamma model far better than any raw-data detector in this comparison matches its own — good model fit compensates for a slow growth rate. A practitioner should therefore read a slow-growth classification as "the mapping from nominal Pfa to threshold is inherently coarse for this family, so calibration accuracy at small Pfa depends unusually heavily on getting the shape fit right," not as "this family will always calibrate badly." The two mechanisms compound; neither substitutes for the other, and a paper — or a detector-selection decision — that reports only one is reporting half the picture.

Reporting practice: an achieved-Pfa or divergence-ratio number without its confidence interval is not a fully specified claim, and the direction of the risk is counter-intuitive. §4 shows the asymmetry directly: this paper's largest, most alarming divergence figures are also its tightest, because a large false-alarm rate produces a large false-pixel count and therefore a narrow Clopper-Pearson interval almost for free. The numbers most likely to be under-scrutinized — a detector reported as "well-calibrated," or as producing "zero false alarms" at some sli/Pfa combination — are exactly the ones with the least statistical support, because good calibration produces few or no false pixels to estimate a rate from. A "zero false alarms" claim with no stated sample size is not evidence of a working detector; §4 measured one concrete case where it corresponded to a 95% upper bound of 10x nominal. Any paper in this research program, this one included, that reports a favorable calibration number should report it with an interval attached; the growth-rate result in §2 explains why the underlying quantity being estimated is so sensitive to the fitted shape and window geometry in the first place, and §4's grid-wide result (median relative width 7.9%, 90th percentile 67.7% on the SSDD grid) shows this is not a rare edge case within this comparison's own data.

Limitations. The reachability-bound framing in this paper concerns Pfa calibration specifically; it says nothing about detection probability or ROC behavior, which is the subject of separate work in this research program. The Clopper-Pearson intervals in §4 treat background pixels as independent Bernoulli trials, which neighboring, overlapping reference windows are not — the true intervals are somewhat wider than reported, in a direction that only strengthens this paper's caution about small-count cells, not its confidence in large ones. The four asymptotic derivations in §2.1 (all but Weibull and Burr XII) are leading-order results with stated, verified error terms, not closed algebraic identities; for K-distribution and Generalized Gamma's v>0 branch specifically, convergence to that leading order is slow (O(log L / L)), so the growth-rate class is established with confidence while the exact constant at any single moderate Pfa is not. The literature search in §5 was targeted, not exhaustive, and is reported as such.

A natural question this result raises for future work is whether nonparametric CFAR families sidestep the problem entirely. Rank-order CFAR detectors (e.g., [2]) determine their detection threshold by exact enumeration of a discrete rank-order statistic rather than by inverting a continuous tail — there is no delta(shape, Pfa) function to have a slow growth rate at all, at the cost of the detector sensitivity such tests are already known to trade away at very low Pfa. Whether that tradeoff is favorable specifically because of the reachability bound identified here, rather than for the previously known reasons alone, was not evaluated in this paper and is worth a dedicated comparison.

VII. CONCLUSION

This paper identified a source of Pfa miscalibration in parametric SAR CFAR detectors that is independent of clutter-model mismatch, estimation noise, and cross-scene variation — all previously documented mechanisms — and is instead a property of the clutter distribution family itself. Every detector considered here reduces to a shared decision rule in which a threshold offset delta(shape, Pfa) is added to a clutter-level term; how fast that offset grows as the requested Pfa shrinks toward zero is fixed by the family's tail type and differs by a full order of growth between families in routine use for SAR clutter. Six such families were derived and verified: two exactly (Weibull, Burr XII) and four as asymptotics cross-checked numerically against their real implementations rather than trusted as algebra, catching one real derivation error in the process (§2.1). The result is a three-class taxonomy — log-log, square-root-log, and linear-log growth — determined by whether the underlying clutter tail is light (Gumbel-type) or heavy (power-law), not by which of the six detectors it happens to be.

This growth-rate property was shown to be a distinct axis from a recently published cross-scene Pfa-divergence result, resolved directly against this project's own controlled measurement grid rather than argued from first principles alone (§3), and every quantitative claim in this paper was attached to an exact confidence interval computed from the appropriate sample size, revealing that the most dramatic-looking numbers in this comparison are also its best-supported, while the most reassuring ones frequently are not (§4). No collision with this specific result was found in a targeted, though not exhaustive, search of the accessible literature (§5).

The practical consequence for SAR CFAR practice is not that any detector family is unusable, nor that any achievable Pfa is bounded below — no such bound exists, and this paper has been explicit throughout that none of its six growth-rate formulas saturate. The consequence is narrower and more actionable: a detector family's growth-rate class should be reported alongside any claim about its calibration accuracy at small nominal Pfa, because that class determines how much statistical and numerical precision the rest of the system — shape estimation, fixed-point hardware quantization, sample size — needs to supply before turning the nominal-Pfa dial produces any real effect at all. A slow-growing family is not a bad family; it is a family that demands more of everything else around it, and a calibration claim made about one without saying so is an incomplete claim.

REFERENCES

[1] X. Meng, "Wilcoxon nonparametric CFAR scheme for ship detection in SAR image," IEEE J. Sel. Topics Appl. Earth Observ. Remote Sens., vol. 18, pp. 5360–5377, 2025, doi: 10.1109/JSTARS.2025.3533140.

[2] A. Sarma and D. W. Tufts, "Rank-order adaptive CFAR: Performance bounds and efficient implementation," IEEE Trans. Aerosp. Electron. Syst., vol. 49, no. 4, pp. 2211–2224, Oct. 2013.

[3] S. Redjem and M. Sahed, "Noncoherent parameter-free GM-CFAR processor for multipulse detection in heavy-tailed Pareto-distributed clutter," in Proc. 2nd Int. Conf. Adv. Technol. Electron. Electr. Eng. (ICATEEE), 2025.

[4] M. E. A. Bouchelaghem, A. A. Meddah, and N. Nouar, "Hybrid algorithm for CFAR detection in non-homogeneous environments," in Proc. 2nd Int. Conf. Adv. Electron. Control Commun. Syst. (ICAECCS), 2025.

[5] J. O. Hinz, M. Holters, U. Zölzer, A. Gupta, and T. Fickenscher, "Presegmentation-based adaptive CFAR detection for HFSWR," in Proc. 2012 IEEE Radar Conf., 2012, pp. 665–670, doi: 10.1109/RADAR.2012.6212222.

[6] J. H. J. Iinatti, "On the threshold setting principles in code acquisition of DS-SS signals," IEEE J. Sel. Areas Commun., vol. 18, no. 1, pp. 62–72, Jan. 2000.

[7] W. Liu, W. Xie, R. Li, F. Gao, X. Hu, and Y. Wang, "Adaptive detection in the presence of signal mismatch," J. Syst. Eng. Electron., vol. 26, no. 1, pp. 38–43, Feb. 2015.

[8] N. Ž. Petrović, M. Petrović, and V. Milovanović, "From front-end to perception: A Chisel generator for low-latency FMCW radar signal processing," in Proc. 33rd Telecommun. Forum (TELFOR), 2025, doi: 10.1109/TELFOR69190.2025.11314346.